# Arduino Projects

Project log — compiled from chat

# Project 1: LED Blink Circuit (Tinkercad)

A simple circuit that blinks an LED on and off every second using an Arduino Uno, simulated in Tinkercad.

## Components used

- Arduino Uno R3

- 1 x LED

- 1 x Resistor (220–330 ohm)

## Build steps

1. Open Tinkercad and start a new circuit

Go to tinkercad.com and sign in (or create a free account). From the dashboard, click ‘Circuits’ in the left sidebar, then click ‘Create new Circuit’. This opens a blank workspace where you can drag and drop components.

2. Add an Arduino Uno R3

In the components search bar on the right, type ‘Arduino Uno’ and drag the ‘Arduino Uno R3’ board onto the canvas. This is the brain of the circuit.

3. Add an LED

Search for ‘LED’ and drag one onto the canvas. The longer leg is the anode (+), the shorter/flat-side leg is the cathode (−).

4. Add a resistor

Search for ‘Resistor’ and drag one onto the canvas. Set its value to 220–330 ohm to protect the LED from excess current.

5. Wire the anode to a digital pin

Connect the LED anode to digital pin 13 on the Arduino (not 5V — a digital pin lets code control the blink).

6. Wire the cathode through the resistor to GND

Connect the LED cathode to one leg of the resistor, then the other resistor leg to a GND pin on the Arduino.

7. Open the code editor and add the blink sketch

Click ‘Code’, switch to ‘Text’ mode, and replace the default code with the blink sketch below.

8. Start the simulation

Click ‘Start Simulation’. The LED will turn on for 1 second, off for 1 second, and repeat — a true blink.

## Arduino sketch (blink code)

void setup() { pinMode(13, OUTPUT); } void loop() { digitalWrite(13, HIGH); delay(1000); digitalWrite(13, LOW); delay(1000); }

Adjust the delay(1000) values (in milliseconds) to change the blink speed.

## Circuit wiring summary

- LED anode → Arduino digital pin 13

- LED cathode → resistor → Arduino GND

Note: wiring the anode directly to 5V instead of a digital pin gives a constant-on LED, not a blink — the blink requires code controlling a digital pin.

## Reference image style

For future circuit builds in this project, circuit images should be generated in a realistic Tinkercad-screenshot style (actual board and component look, colored jumper wires) rather than an abstract schematic — matching the reference image shared in chat, with only the specific requested changes applied each time.